# Lab 1: Ethernet-Based Connection of Network Devices

## Step 1: Making an Ethernet Cable Using RJ45 Connectors

### Required Equipment:

- Ethernet cable (Cat5e / Cat6)

- RJ45 connectors

- Crimping tool

- Wire stripper

- Network cable tester

### Procedure:

#### A. Straight-Through Ethernet Cable

- Measure and cut the Ethernet cable to the needed length.

- Remove approximately one inch of the outer insulation from both ends.

- Separate and straighten the wire pairs, then arrange them according to the T568B standard:

- Pin configuration (T568B):

1.White-Orange, 2. Orange, 3. White-Green, 4. Blue, 5. White-Blue, 6. Green, 
7. White-Brown, 8. Brown

- Trim all wires evenly so they align properly.

- Insert the wires fully into the RJ45 connector while maintaining the correct order.

- Use a crimping tool to firmly attach the connector.

- Repeat the same process on the other end using the identical wiring sequence.

- Verify the cable using a cable tester.

#### B. Crossover Ethernet Cable

- Cut the Ethernet cable to the required size and strip the outer jacket from both ends.

- Arrange one end of the cable following the T568A standard:

- Pin configuration (T568A):

1.White-Green, 2. Green, 3. White-Orange, 4. Blue, 5. White-Blue, 6. Orange, 
7. White-Brown, 8. Brown

- Arrange the other end using the T568B wiring standard.

- Pin configuration (T568B):

1.White-Orange, 2. Orange, 3. White-Green, 4. Blue, 5. White-Blue, 6. Green, 
7. White-Brown, 8. Brown

- Ensure all wires are trimmed evenly before insertion.

- Insert the wires into the RJ45 connectors carefully.

- Crimp both ends securely using the crimping tool.

- Test the completed cable using a cable tester to confirm proper crossover connections.

## Cable Testing Procedure

- Connect one end of the Ethernet cable to the main unit of the cable tester.

- Attach the opposite end to the remote unit.

- Switch on the tester and monitor the LED indicators.

- Straight-through cables should display continuity from pins 1 to 8 in sequence.

- Crossover cables should show crossed pairs (1↔3 and 2↔6Output: Straight

## Output: Straight

## Output: Crossover

## Result

The cable tester confirms correct wiring with no open or short circuits. The observed outputs for straight-through and crossover cables are documented in the respective image folders.